PUBLISHED BY THE MARK TAYLOR CANDLE SHORE BLOG

# CHAPTER 4

# iPhone 3G S Get Started

# IPHONE 3G S

## (Edited/Formatted  by Mark Taylor)

The following information has been compiled, edited, and formatted for those reading with either a screen reader, screen magnifier, or refreshable Braille device.

# Get Started

## What You Need

To avoid injury, read all operating instructions in this guide and safety information in the iPhone Important Product Information Guide before using iPhone.

To use iPhone, you need:

A wireless service plan with a carrier that provides iPhone service in your area

A Mac or a PC with a USB 2.0 port and one of the following operating systems:

Mac OS X version 10.4.11 or later; version 10.5.7 or later is required for syncing Notes and for using iPhone as a modem

Windows XP Home or Professional with Service Pack 2 or later

Windows Vista Home Premium, Business, Enterprise, or Ultimate edition

Display resolution on your computer set to 1024 x 768 or higher

iTunes 8.2 or later, available at www.itunes.com/download

QuickTime 7.6.2 or later (for playing videos recorded by iPhone 3G S on your computer)

An iTunes Store account (for purchases from the iTunes Store or App Store)

An Internet connection for your computer (broadband recommended)

## Activating iPhone

Before you can use any of iPhone’s features, you must activate iPhone by signing up for a service plan with an iPhone service carrier in your area and registering iPhone with the network.

Your iPhone may have been activated at the time of purchase. If it isn’t activated, contact your iPhone retailer or cellular service provider.

For more information about iPhone, go to www.apple.com/iphone.

## Installing the SIM Card

If your SIM card was not preinstalled, you must install the SIM card before you can use iPhone.

Install the SIM card:

Insert the end of the SIM eject tool into the hole on the SIM tray.

Press firmly and push it straight in until the tray pops out. If you don’t have a SIM eject tool, you can use the end of a paper clip.

Pull out the SIM tray and place the SIM card in the tray.

The angled corner of the SIM ensures that the card fits only the correct way in the tray.

With the tray aligned, carefully replace the SIM tray containing the SIM card in iPhone.

## Registering iPhone

Registering iPhone with iTunes enables iTunes to identify your iPhone when it’s connected to your computer and help you manage its contents. You can then sync information with your computer and media from iTunes, and create backups of iPhone’s contents and settings. You can create an iTunes Store account, or specify an existing account, to enable purchases with iPhone. iTunes also records iPhone’s serial number in the event you need it for service or in case of loss.

Register iPhone:

Download and install the latest version of iTunes from www.itunes.com/download.

Connect iPhone to a USB 2.0 port on your Mac or PC using the cable that came with iPhone.

Follow the onscreen instructions in iTunes to register iPhone and sync iPhone with your contacts, calendars, and bookmarks on your computer, and with music, video, and other content from your iTunes library.

In the Set Up Your iPhone screen, select “Automatically sync contacts, calendars and bookmarks” to configure those items to sync automatically when you connect iPhone to your computer. Or, you can customize your sync settings in iTunes.

## Setting Up iPhone Using VoiceOver

If you are sight imparied, VoiceOver may help you set up iPhone without outside assistance. VoiceOver  describes aloud what appears on the screen so that you can use iPhone without seeing it. When you connect iPhone to your computer, iTunes detects when you’re using a compatible screen reader on your computer, such as Voice Over on a Mac or GW-Micro Window-Eyes on a PC, and automatically enables VoiceOver on iPhone. Or a sighted user can enable VoiceOver on iPhone for you, using the Accessibility settings. See Using VoiceOver.

## About Syncing

With iPhone, you can have easy access to important information on your computer, including your contacts, calendars, and even your browser bookmarks, as well as to music, video, and other content in your  iTunes library. Syncing copies information and content between iPhone and your computer, keeping it current and up-to-date, whether you make changes on iPhone or on your computer.

iTunes lets you specify the information and content on your computer that you want to sync with iPhone. By default, iTunes syncs automatically with iPhone whenever you connect iPhone to your computer.

You can also set up iPhone to sync your MobileMe and Microsoft Exchange accounts, over the air. See Setting Up Accounts.

## Syncing with iTunes

You can set iTunes to sync any or all of the following:

Contacts—names, phone numbers, addresses, email addresses, and more

Calendars—appointments and events

Email account settings

Webpage bookmarks

Notes (requires Mac OS X version 10.5.7 on a Mac)

Ringtones

Music and audiobooks

Photos

Podcasts

Movies, TV shows, and music videos

Applications downloaded from the iTunes Store

You can adjust sync settings whenever iPhone is connected to your computer.

Ringtones, music, audiobooks, podcasts, video content, and applications are synced from your iTunes library. If you don’t already have content in iTunes, the iTunes Store (available in some countries) makes it easy to preview and download content to iTunes. You can also add music to your iTunes library from your CDs. To learn about iTunes and the iTunes Store, open iTunes and choose Help _ iTunes Help.

Contacts, calendars, notes, and webpage bookmarks are synced with applications on your computer. Contacts and calendars are synced both ways between your computer and iPhone. New entries or changes you make on iPhone are synced to your computer, and vice versa. Notes and webpage bookmarks are also synced both ways. Photos can be synced from an application or from a folder.

Email account settings are synced only from your computer’s email application to iPhone. This allows you to customize your email accounts on iPhone without affecting email account settings on your computer.

NOTE: You can also set up email accounts directly on iPhone. See Setting Up Accounts.

Purchases you make on iPhone in the iTunes Store or the App Store are synced back to your iTunes library. You can also purchase or download content and applications directly from the iTunes Store on your computer, and then sync them to iPhone.

You can set iPhone to sync with only a portion of what’s on your computer. For example, you might want to sync only a group of contacts from your address book, or only unwatched video podcasts.

IMPORTANT: You should be logged in to your own computer user account before connecting iPhone.

Set up iTunes syncing:

Connect iPhone to your computer, and open iTunes (if it doesn’t open automatically).

In iTunes, select iPhone in the sidebar.

Configure the sync settings in each of the settings panes.

Click Apply in the lower-right corner of the screen.

By default, “Automatically sync when this iPhone is connected” is selected.

## iPhone Settings Panes in iTunes

The following sections provide an overview of each of the iPhone settings panes. For more information, open iTunes and choose Help _ iTunes Help.

Summary Pane

Select “Automatically sync when this iPhone is connected” to have iTunes sync iPhone automatically whenever you connect it to your computer. Deselect this option if you want to sync only by clicking the Sync button in iTunes. For more information about preventing automatic syncing, see Preventing Automatic Syncing.

Select “Sync only checked songs and videos” if you want iTunes to skip unchecked items in your iTunes library when syncing.

Select “Manually manage music and videos” to turn off automatic syncing in the Music and Video settings panes. See Manually Managing Content.

Select “Encrypt iPhone backup” if you want to encrypt the information stored on your computer when iTunes makes a backup. Encrypted backups are indicated by a padlock icon, and a password is required to restore the information to iPhone. See Creating Backups.

Click Configure Universal Access to turn on iPhone’s Accessibility features. See Accessibility.

Info Pane

The Info pane lets you configure the sync settings for your contacts, calendars, email accounts, and web browser.

Contacts

Sync contacts with applications such as Mac OS X Address Book, Yahoo! Address Book, and Google Contacts on a Mac, or with Yahoo! Address Book, Google Contacts, Windows Address Book (Outlook Express), Windows Vista Contacts, or Microsoft Outlook 2003 or 2007 on a PC. (On a Mac, you can sync contacts with multiple applications. On a PC, you can sync contacts with only one application at a time.)

If you sync with Yahoo! Address Book, you only need to click Configure to enter your new login information when you change your Yahoo! ID or password after you’ve set up syncing.

Calendars

Sync calendars from applications such as iCal on a Mac, or from Microsoft Outlook 2003 or 2007 on a PC. (On a Mac, you can sync calendars with multiple applications. On a PC, you can sync calendars with only one application at a time.)

Mail Accounts

Sync email account settings from Mail on a Mac, and from Microsoft Outlook 2003 or 2007 or Outlook Express on a PC. Account settings are transferred only from your computer to iPhone. Changes you make to an email account on iPhone don’t affect the account on your computer.

NOTE: The password for your Yahoo! email account isn’t saved on your computer, so it can’t be synced and must be entered on iPhone. In Settings, choose “Mail, Contacts, Calendars,” tap your Yahoo! account, and enter the password.

Web Browser

You can sync bookmarks on iPhone with Safari on a Mac, or with Safari or Microsoft Internet Explorer on a PC.

Notes

Sync notes in the Notes application on iPhone with notes in Mail on a Mac or with Microsoft Outlook on a PC. (On a Mac, you must have Mac OS version 10.5.7 or later installed to sync notes.)

Advanced

These options let you replace the information on iPhone with the information on your computer during the next sync.

Ringtones Pane

Use the Ringtones pane to select the ringtones you want to sync to iPhone. To learn how to create ringtones in iTunes, see Creating Ringtones.

Music, Podcasts, and Video Panes

Use these panes to specify the media you want to sync. You can sync all music, podcasts, and videos, or select the playlists and specific podcasts and videos you want on iPhone.

If you want to watch rented movies on iPhone, transfer them to iPhone using the Video  pane in iTunes.

If there’s not enough room on iPhone for all the media you’ve specified, iTunes asks if you want to create a special playlist. iTunes creates the playlist for you, and sets it to sync with iPhone.

Photos Pane

You can sync photos with iPhoto 4.0.3 or later, or Aperture on a Mac; or with Adobe Photoshop Album 2.0 or later, or Adobe Photoshop Elements 3.0 or later on a PC. You can also sync photos in any folder on your computer that contains images.

Applications Pane

Use the Applications pane to specify which App Store applications you want synced on iPhone. Any applications downloaded directly to iPhone are automatically backed up to your iTunes library when you sync. If you manually delete an application on iPhone, you can reinstall it from this pane as long as it was previously synced.

## Preventing Automatic Syncing

You may want to prevent iPhone from syncing automatically when you connect iPhone to a computer other than the one you usually sync with.

Turn off automatic syncing for iPhone:

Connect iPhone to your computer. In iTunes, select iPhone in the sidebar, then click the Summary tab. Deselect “Automatically sync when this iPhone is connected.”  You can always sync by clicking the Sync button.

Prevent automatic syncing for all iPhones and iPods:

In iTunes choose iTunes _ Preferences (on a Mac) or Edit _ Preferences (on a PC), click Devices, and select “Disable automatic syncing for all iPhones and iPods.”

If this checkbox is selected, iPhone won’t sync automatically, even if “Automatically sync when this iPhone is connected” is selected in the Summary pane.

Prevent automatic syncing one time, without changing settings:

Open iTunes. Then, as you connect iPhone to your computer, press and hold Command-Option (on a Mac) or Shift-Control (on a PC) until you see iPhone appear in the sidebar.

Sync manually:

In iTunes, select iPhone in the sidebar, then click Sync in the lower-right corner of the window. Or, if you’ve changed any sync settings, click Apply.

## Setting Up Accounts

iPhone works with MobileMe, Microsoft Exchange, and many of the most popular email systems.  MobileMe and Microsoft Exchange provide not only email, but contact and calendar information that can be synced to iPhone automatically, over the air. MobileMe can also sync your bookmarks on iPhone with Safari on a Mac, or with Safari or Microsoft Internet Explorer on a PC. You set up MobileMe, Exchange, and other email accounts directly on iPhone.

iPhone uses the Exchange ActiveSync protocol to sync email, calendars, and contacts with the following versions of Microsoft Exchange:

Exchange Server 2003 Service Pack 2

Exchange Server 2007 Service Pack 1

For many of the popular email accounts, iPhone automatically enters most of the settings for you.

If you don’t already have an email account, you can get a free account online at www.yahoo.com, www.google.com, or www.aol.com. You can also try MobileMe, free for 60 days. Go to www.me.com.

Set up an account on iPhone:

From the iPhone Home screen, tap Settings.

Tap “Mail, Contacts, Calendars,” then tap Add Account.

Tap an account type:

Microsoft Exchange

MobileMe

Google email

Yahoo! Mail

AOL

Other

Enter your account information and tap Save.

Your service provider or system administrator can provide the account settings you need.

Exchange accounts:  Enter your complete email address, domain (optional), user name, password, and a description. The description can be whatever you like.

iPhone supports Microsoft’s Autodiscovery service, which uses your user name and password to determine the address of the Exchange server. If the server’s address can’t be determined, you’re asked to enter it. (Enter the complete address in the Server field.) Once you connect to the Exchange server, you may be prompted to change your passcode to match the policies set on the server.

If you’re setting up a MobileMe or Microsoft Exchange account, tap the items you want to use on iPhone—mail, contacts, calendars, and bookmarks (MobileMe only).

MobileMe includes the Find My iPhone feature that helps you locate iPhone if it’s been lost or stolen, and remotely erase the information on iPhone if necessary. See Security Features. If you enable Find My iPhone for your MobileMe account, make sure that Push is turned on for Fetch New Data. In Settings, choose “Mail, Contacts, Calendars” _ Fetch New Data, and if Push is off, tap to turn it on.

For Exchange accounts, you can set how many days of email you want to sync to iPhone.

If you already have contact or calendar information on iPhone when you enable the account, iPhone asks what you want to do with existing information on iPhone. You can choose to merge the existing data into your MobileMe or Exchange account. If you don’t merge the data, you can choose to keep it in a separate account on iPhone (the account is labeled “On My iPhone”), or delete it. Existing bookmarks can only be merged or deleted.

Similarly, when you disable contacts or calendars in an MobileMe or Exchange account, or if you delete an account, iPhone asks if you want to keep information or delete it.

You can add contacts using an LDAP account if your company or organization supports it. See Adding Contacts.

You can subscribe to CalDAV or iCal (.ics) calendars. See Subscribing to Calendars.

## Push Accounts

MobileMe, Microsoft Exchange, and Yahoo! Mail are known as “push” accounts. When new information is available, such as the arrival of an email message, the information is automatically delivered (pushed) to iPhone. (In contrast, “fetch” services require your email software to periodically check with your service provider to see if new messages have arrived, and then request them to be delivered.) MobileMe and Exchange also use push to sync your contacts, calendars, and bookmarks (MobileMe only) if you have those items selected in the account settings.

Synced information is transferred automatically over your wireless connection; you don’t need to connect iPhone to your computer to sync. If you don’t have a cellular connection, iPhone can receive pushed data over a Wi-Fi connection when iPhone is awake (the screen is on, or iPhone is connected to your computer or a power adapter).

## Installing Configuration Profiles

If you’re in an enterprise environment, you may be able to set up accounts and other items on iPhone by installing a configuration profile. Configuration profiles provide a way for system administrators to quickly set up your iPhone to work with the information systems at your company, school, or organization. For example, a configuration profile might set up your iPhone to access the Microsoft Exchange servers at work, enabling iPhone to access your Exchange email, calendars, and contacts.

Configuration profiles can configure multiple settings on iPhone all at once. For example, a configuration profile can set up your Microsoft Exchange account, VPN account, and certificates for secure access to your company’s network and information. A configuration profile may turn on the Passcode Lock, requiring you to create and enter a passcode for using iPhone.

Your system administrator may distribute configuration profiles by email or by putting them on a secure webpage. A system administrator may also install a configuration profile directly on iPhone for you.

Install a configuration profile:

Using iPhone, open the email message or download the configuration profile from the website provided by your system administrator.

When the configuration profile opens, tap Install.

Enter passwords and other information as requested.

IMPORTANT: You may be asked to verify that the configuration profile is trusted. If in doubt, consult your system administrator before installing a configuration profile.

Settings provided by a configuration profile cannot be altered. If you want to change these settings, you must first remove the configuration profile or install an updated configuration profile.

Remove a profile:

In Settings, choose General _ Profile, then select the configuration profile and tap Remove.

Removing a configuration profile deletes the settings and all other information installed by the configuration profile from iPhone.

## Disconnecting iPhone from Your Computer

Unless iPhone is syncing with your computer, you can disconnect it from your computer at any time.

When iPhone is syncing with your computer, iPhone shows “Sync in progress.” If you disconnect iPhone before it finishes syncing, some data may not get transferred. When iPhone finishes syncing, iTunes shows “iPhone sync is complete.”

Cancel a sync:

Drag the slider on iPhone.

If you get a call during a sync, the sync is canceled automatically and you can unplug iPhone to answer the call. Connect iPhone after the call to finish syncing.